Multi/Pak 95

Furnish and install as shown on the plans Multi/Pak® 95—flat-top fin-tube enclosures with elements, required mounting components and all accessories as manufactured by Slant/Fin Corporation. 

Cover shall be of two interlocking sections: a top cover and front cover. Top cover will be common to onetier and two-tier models. (A knob-actuated damper shall be provided.) Top of enclosure shall consist of stamped grille pre-painted and with pencil-proof air discharge louvers. Front skirt shall interlock with top panel to form lateral decorative panel. Front skirt shall be removable. 

Interlocking top cover and front cover shall both be fabricated from 18-gauge steel in Nu-White baked enamel or as custom matched as per specification.  Brackets and hangers have electro-galvanized finish.

Fin-tube element(s) shall be Model___________.   Provide lengths and capacities as scheduled on plans.  Furnish required channeled bracket-hanger assemblies with heavy flag brace for rigid front skirt and element support. Provide all required accessories for complete installation. Cover accessories shall be telescopic and shall match enclosure color.

